Module 16: Documents and Records

| Purpose | To provide the participants with information for managing documents and records. |
| --- | --- |
| Module Time (suggested) | 95 minutes: 80 minutes for presentation 15 minutes for activities (2) |
| Learning Objectives | At the end of this module, participants will be able to: - explain the difference between documents and records; - describe the hierarchy of documents and the role of each level; - outline the content that should be included in a standard operating procedure; - explain the important steps, or elements, of a laboratory document management system; - outline the contents of a quality manual; - describe methods and tools to properly store documents and records. |
| Test Questions (suggested) | Suggested questions for this module (with answers bolded) include: - Written policies, processes, and procedures are necessary for all of the following reasons EXCEPT: - verbal instructions may be misunderstood - laboratory inspectors require it - they record data or information about what happened, and are not revised or modified - reproducibility of results is improved - Procedures: - tell “what to do”, and define the overall intentions and directions of the organization - tell “how it happens”, and can generally be represented in a flow chart indicating how events should occur over a period of time - tell “how to do it”, and show the step-by-step instructions that laboratory staff should meticulously follow to produce accurate and consistent results - The following is an example of a document: - a Quality Manual - a log of patient results - a chart showing the temperatures of a refrigerator over the last month - a graph plotting quality control results |
| Module Overview | The following table suggests an agenda for organizing the module. Each trainer can modify it, as well as select and customize the available resources. |

| Step | Time | Topics | Contents | Available resources |
| --- | --- | --- | --- | --- |
| 1 | 5 min | Introduction to documents and records | Content sheet 16-1 | Presentation 16, Slides 1-4 Activity sheet 16-1 |
| 2 | 15 min | Overview of documents | Content sheet 16-2 | Presentation 16, Slides 5-16 |
| 3 | 10 min | The Quality Manual | Content sheet 16-3 Annex 16-A, Annex 16-B | Presentation 16, Slides 18-22 Activity sheet 16-2 |
| 4 | 15 min | Standard Operating Procedures (SOPs) | Content sheet 16-4 Annex 16-C, Annex 16-D | Presentation 16, Slides 23-34 |
| 5 | 15 min | Document control | Content sheet 16-5 | Presentation 16, Slides 35-42 |
| 6 | 10 min | Overview of records | Content sheet 16-6 | Presentation 16, Slides 43-47 |
| 7 | 10 min | Storing documents and records | Content sheet 16-7 | Presentation 16, Slides 48-51 |
| 8 | 15 min | Summary | Content sheet 16-8 | Presentation 16, Slides 52-56 Activity sheet 16-1 |
| Optional Material on the CD-ROM |  |  |  |  |
| Structure and contents of a quality manual |  |  | Content sheet 16-9 | Presentation 16, Optional Slides 1-23 |
| Developing SOPs |  |  |  | Activity sheet 16-3 |
| Visual aids examples |  |  |  | Presentation 16, Optional Slides 1-6 |
| Developing a document control program |  |  |  | Activity sheet 16-4 |

Material/Equipment Checklist

- PowerPoint slides or transparencies

- Overhead projector or computer with an LCD projector

- Prepared flipchart, white board, or chalk board

- Paper cards, markers, and tape

- Additional handouts as required.