Module 10: Assessment: External Quality Assessment

| Purpose |  |  | To provide the participants with aspects of External Quality Assessment (EQA) that contribute to quality laboratory practices. |  |  |
| --- | --- | --- | --- | --- | --- |
| Module Time (suggested) |  |  | 90 minutes: 55 minutes for presentation 35 minutes for activities (2) |  |  |
| Learning Objectives |  |  | At the end of this module, participants will be able to: - discuss the importance of an EQA program in improving the quality of laboratory test results; - describe at least three EQA methods and the advantages and disadvantages of each; - outline a method to investigate an unacceptable test result from an EQA sample. |  |  |
| Test Questions (suggested) |  |  | Suggested test questions (with answers highlighted) for this module include: - All of the following are EQA methods EXCEPT: - proficiency testing - on-site evaluations from an external agency - quality control of quantitative testing - retesting or rechecking samples exchanged between laboratories - Your laboratory received an unstained blood slide from an EQA provider with a request to look for malaria parasites. Which of the following choices would you do to insure this sample is reported correctly? - stain the slide with fresh reagents and give to the supervisor to examine - stain the slide with fresh reagents and ask all employees that do malaria smears to examine it and compare answers - stain and read the slide using your best employees, then call neighboring laboratories to confirm results - examine the slide in the same way the laboratory routinely handles patients samples - EQA is an important component of quality management because: - it guarantees that the daily results of patients’ samples are accurate - it always monitors the pre-examination, examination, and post-examination phases of the workflow path - it provides information to consumers about the quality of testing for a particular EQA sample - it is an objective way to measure the quality of testing in a laboratory |  |  |
| Module Overview |  |  | The following table suggests an agenda for organizing the module. Each trainer can modify it, as well as select and customize the available resources. |  |  |
| Step | Time | Core Topics |  | Contents | Available resources |
| 1 | 25 min | Introduction |  |  | Presentation 10, Slides 1-3 Activity sheet 10-1 |
| 2 | 10 min | Overview of external quality assessment (EQA) |  | Content sheet 10-1 | Presentation 10, Slides 4-10 |
| 3 | 30 min | Proficiency testing (PT) |  | Content sheet 10-2 | Presentation 10, Slides 11–16 Activity sheet 10-2 |
| 4 | 10 min | Other EQA methods |  | Content sheet 10-3 Annex 10-A Annex 10-B | Presentation 10, Slides 17-22 |
| 5 | 5 min | Comparison of EQA methods |  | Content sheet 10-4 Annex 10-C | Presentation 10, Slide 23 |
| 6 | 5 min | Managing EQA in the laboratory |  | Content sheet 10-5 | Presentation 10, Slides 24-27 |
| 7 | 5 min | Summary |  | Content sheet 10-6 | Presentation 10, Slides 28-30 |
| Optional Material on the CD-ROM |  |  |  |  |  |
| Organizing a proficiency testing program |  |  |  | Content sheet 10-7 | Presentation 10, Optional Slides 1-13 |

Material/Equipment Checklist

- PowerPoint Slides or transparencies

- Overhead projector or computer with an LCD projector

- Prepared flipchart, white board, or chalk board

- Paper cards, markers, and tape

- Additional handouts as required.