Activity 7-1: Calculation of Mean and Standard Deviation

| Purpose | To practice calculating the mean and standard deviation (SD) of a set of data to use for establishing control ranges. |
| --- | --- |
| Suggested Time | 15 minutes |
| Instructional Method | Calculate the mean and SD using Annex 7-A and the two Standard Deviation Worksheets provided. |

| Standard Deviation Worksheet 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Method: Glucose |  |  | Sample: Pooled Serum, Level 1 Control |  |  |  |
| Day | Observation (x) |  | Difference (x-mean) |  | Difference (x-mean) Squared |  |
| 1 | 98 |  |  |  |  |  |
| 2 | 95 |  |  |  |  |  |
| 3 | 103 |  |  |  |  |  |
| 4 | 102 |  |  |  |  |  |
| 5 | 100 |  |  |  |  |  |
| 6 | 105 |  |  |  |  |  |
| 7 | 103 |  |  |  |  |  |
| 8 | 95 |  |  |  |  |  |
| 9 | 92 |  |  |  |  |  |
| 10 | 93 |  |  |  |  |  |
| 11 | 108 |  |  |  |  |  |
| 12 | 98 |  |  |  |  |  |
| 13 | 98 |  |  |  |  |  |
| 14 | 101 |  |  |  |  |  |
| 15 | 103 |  |  |  |  |  |
| 16 | 104 |  |  |  |  |  |
| 17 | 100 |  |  |  |  |  |
| 18 | 100 |  |  |  |  |  |
| 19 | 92 |  |  |  |  |  |
| 20 | 91 |  |  |  |  |  |
| Sum: |  |  |  |  |  |  |
| Mean: |  |  |  |  |  |  |
| Variance: |  |  |  |  |  |  |
| SD: |  |  |  |  |  |  |
|  | Lower Limit |  | Upper Limit |  |  |  |
| 1 SD |  |  |  |  |  |  |
| 2 SD |  |  |  |  |  |  |
| 3 SD |  |  |  |  |  |  |
| Standard Deviation Worksheet 2 |  |  |  |  |  |  |
| Method: Glucose |  |  |  | Sample: Pooled Serum, Level 2 Control |  |  |
| Day |  | Observation (x) |  | Difference (x-mean) |  | Difference (x-mean) Squared |
| 1 |  | 293 |  |  |  |  |
| 2 |  | 279 |  |  |  |  |
| 3 |  | 277 |  |  |  |  |
| 4 |  | 282 |  |  |  |  |
| 5 |  | 279 |  |  |  |  |
| 6 |  | 279 |  |  |  |  |
| 7 |  | 281 |  |  |  |  |
| 8 |  | 286 |  |  |  |  |
| 9 |  | 276 |  |  |  |  |
| 10 |  | 282 |  |  |  |  |
| 11 |  | 265 |  |  |  |  |
| 12 |  | 267 |  |  |  |  |
| 13 |  | 273 |  |  |  |  |
| 14 |  | 289 |  |  |  |  |
| 15 |  | 270 |  |  |  |  |
| 16 |  | 276 |  |  |  |  |
| 17 |  | 269 |  |  |  |  |
| 18 |  | 280 |  |  |  |  |
| 19 |  | 285 |  |  |  |  |
| 20 |  | 279 |  |  |  |  |
| Sum: |  |  |  |  |  |  |
| Mean: |  |  |  |  |  |  |
| Variance: |  |  |  |  |  |  |
| SD: |  |  |  |  |  |  |
|  |  | Lower Limit |  | Upper Limit |  |  |
| 1 SD |  |  |  |  |  |  |
| 2 SD |  |  |  |  |  |  |
| 3 SD |  |  |  |  |  |  |

Activity 7-2: Creation of Levey-Jennings Charts

| Purpose | To create Levey-Jennings Charts to monitor examination results by visualizing daily control data. |
| --- | --- |
| Suggested Time | 15 minutes |
| Instructions | Use the mean and SDs calculated in Activity 7-1 for Level 1 and Level 2 Controls to make two Levey-Jennings Charts on graph paper. - Draw a line in the middle of one section of the graph paper to represent the mean of the Level 1 control. Write the value of the mean on the left of the line. - Mark off plus (+) and minus (-) 3 SDs at appropriate intervals on the graph, then draw a horizontal line through each, and write the values beside each line. - Label the x-axis with days, runs, or other interval used to run QC. Divide the x-axis into equal spaces and label with time intervals, for example, days. - Label the chart with the name of the test and the control’s lot number. - Repeat for the Level 2 control. - Plot the data provided in Table 1 on the respective Levey-Jennings charts. |