Module 2: Facilities and Safety

| Purpose | To provide the participants with information that contributes to quality laboratory practices and a safe working environment. This module does not intend to provide full knowledge on biosafety, biosecurity, and biochemical issues. |
| --- | --- |
| Module Time (suggested) | 100 minutes: 75 minutes for presentation 25 minutes for activities (3) |
| Learning Objectives | At the end of this module, participants will be able to: - relate how facility design impacts the efficiency and safety of laboratory workers; - describe practices to prevent or reduce risks; - list personal protective equipment (PPE) that should be used routinely by laboratory workers; - explain general safety requirements for the laboratory; - describe steps to take in response to emergencies such as biological or chemical spills, or laboratory fires. |
| Test Questions (suggested) | Suggested test questions (with answers bolded) for this module include: - Why is it important to know the pathway of the sample through the laboratory? - to make sure it doesn’t get lost - to identify where improvements in laboratory design may be needed - to assess laboratory workload - to identify staff that need additional training - The main routes in which chemicals enter the body are (check all that apply): - inhalation - absorption - percutaneous inoculation - ingestion 3. The main sources of contamination within diagnostic laboratories are: - air conditioning vents - aerosols - animal cages - sick employees |
| Module Overview (suggested) | The following table suggests an agenda for organizing the module. Each trainer can modify it, as well as select and customize the available resources. |

| Step | Time | Topics | Contents | Available resources |
| --- | --- | --- | --- | --- |
| 1 | 15 min | Introduction |  | Presentation 2, Slides 1-3 Activity sheet 2-1 |
| 2 | 5 min | Facilities and safety overview | Content sheet 2-1 | Presentation 2, Slides 4-7 |
| 3 | 15 min | Laboratory design | Content sheet 2-2 | Presentation 2, Slides 8-15 Activity sheet 2-2 |
| 4 | 10 min | Geographic or spatial organization | Content sheet 2-3 | Presentation 2, Slides 16-20 |
| 5 | 10 min | Physical aspects of premises and rooms | Content sheet 2-4 | Presentation 2, Slides 21-24 |
| 6 | 15 min | Safety management program | Content sheet 2-5 | Presentation 2, Slides 25-29 Activity sheet 2-3 |
| 7 | 15 min | Identification of risks | Content sheet 2-6 Annex 2-A Annex 2-B | Presentation 2, Slides 30-43 |
| 8 | 5 min | Personal protective equipment | Content sheet 2-7 | Presentation 2, Slides 44-45 |
| 9 | 5 min | Emergency management and first aid | Content sheet 2-8 | Presentation 2, Slides 46-48 |
| 10 | 5 min | Summary | Content sheet 2-9 | Presentation 2, Slides 49-52 |

Material/Equipment Checklist

- PowerPoint slides or transparencies

- Overhead projector or computer with an LCD projector

- Prepared flipchart, white board, or chalk board

- Paper cards, markers, and tape

- Additional handouts as required.