Module 7: Process Control: Quantitative Quality Control

| Purpose | This module explains how quality control methods are applied to quantitative laboratory examinations, and how to organize a quality control program for quantitative tests. |
| --- | --- |
| Module Time (suggested) | 90 minutes: 60 minutes for the presentation 30 minutes for the activities (2) |
| Learning Objectives | At the end of this module, participants will be able to: - differentiate accuracy and precision; - select control material for a specified examination method; - establish acceptable control limits for a method when only one level of control material is available; - explain the use of a Levey-Jennings chart; - give two examples of rule violations using Westgard Multirule System; - describe how to correct “out of control” problems. |
| Test Questions (suggested) | 1. Calibrators have the following characteristics/functions: - contain a known amount of the analyte being tested - monitor the quality of reagents - monitor the quality of the sample - prevent equipment failure 2. The reproducibility of a value upon repeated measurements is referred to as: - accuracy - precision - bias - error 3. The probability that a given control value will fall within + 2 SD of the established mean is: - 68.2% - 99.7% - 95.5% - 92.5% |
| Module Overview (suggested) | The following table suggests an agenda for organizing the module. Each trainer can modify it, as well as select and customize the available resources. |

| Step | Time | Topics | Contents | Available resources |
| --- | --- | --- | --- | --- |
| 1 | 5 min | Introduction |  | Presentation 7, Slides 1-2 |
| 2 | 5 min | Overview of quality control for quantitative tests | Content sheet 7-1 | Presentation 7, Slides 3-5 |
| 3 | 10 min | Control materials | Content sheet 7-2 | Presentation 7, Slides 6-14 |
| 4 | 35 min | Establishing the value range for the control material | Content sheet 7-3 | Presentation 8, Slides 15-30 Activity sheet 7-1, Answer sheet 7-1 |
| 5 | 20 min | Graphically representing control ranges | Content sheet 7-4 Annex 7-A | Presentation 7, Slides 31-33 Activity sheet 7-2 |
| 6 | 10 min | Interpreting quality control data | Content sheet 7-5 | Presentation 7, Slides 34-39 |
| 7 | 5 min | Using quality control information | Content sheet 7-6 | Presentation 7, Slides 40-42 |
| 8 | 5 min | Summary | Content sheet 7-7 | Presentation 7, Slides 43-45 |
| Optional Material on the CD-ROM |  |  |  |  |
| Westgard Multirule System |  |  | Content sheet 7-8 | Presentation 7, Optional slides 1-11 Activity sheet 7-3 |

Material/Equipment Checklist

- PowerPoint slides or transparencies

- Overhead projector or computer with an LCD projector

- Prepared flipchart, white board, or chalk board

- Paper cards, markers, and tape

- Additional handouts as required.