Module 5: Process Control — Sample Management

| Purpose |  |  | The purpose of this module is to present information that will help participants develop a system for managing samples from the time of collection to the final storage or disposal, in a manner that will ensure sample integrity. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Module Time (suggested) |  |  | 95 minutes: 65 minutes for the presentation 30 minutes for the activities (3) |  |  |  |
| Learning Objectives |  |  | At the end of this activity, participants will be able to: - name sample collection errors that could lead to incorrect laboratory examination results; - list contents that should be included in a handbook designed for people who collect samples off-site; - provide a rationale for rejecting unsatisfactory samples; - describe a system for sample handling, including collection, transport, storage, and disposal; - explain the importance of maintaining sample integrity and assuring that all regulations and requirements are met when transporting samples. |  |  |  |
| - Test Questions (suggested) |  |  | Suggested test questions (with answers bolded) for this module include: 1. One of the most difficult aspects of the sample management process for the laboratory to monitor is: - patient identification - preservation of samples - controlling time of collection of sample - reporting results 2. Which of the following statements is TRUE? - a sample should be always processed, regardless of its condition - a sample should be rejected if the wrong preservative was used - a sample should never be processed if it is only labeled with the patient’s name - a sample with insufficient quantity should always be discarded 3. The best method to use to communicate sample collection procedures to non-laboratory staff is to: - provide all collection sites with a Laboratory Handbook - provide individual training to each person who collects samples - provide training to all nursing supervisors and ensure they oversee the process correctly - provide all stations with the laboratory’s quality manual |  |  |  |
| - Module - Overview |  |  | The following table suggests an agenda for organizing the module. Each trainer can modify it, as well as select and customize the available resources. |  |  |  |
| Step | Time | Topics |  | Contents | Available resources |  |
| 1 | 15 min | Introduction |  |  | Presentation 5, Slides 1-3 Activity sheet 5-1 |  |
| 2 | 5 min | Overview of sample management |  | Content sheet 5-1 | Presentation 5, Slides 4-7 |  |
| 3 | 5 min | The Laboratory handbook |  | Content sheet 5-2 | Presentation 5, Slides 8-9 |  |
| 4 | 25 min | Collection and preservation |  | Content sheet 5-3 Annex 5-A Annex 5-B Annex 5-C Annex 5-D | Presentation 5, Slides 10-17 Activity sheet 5-2 |  |
| 5 | 20 min | Sample processing |  | Content sheet 5-4 | Presentation 5, Slides 18-24 Activity sheet 5-3 |  |
| 6 | 5 min | Sample storage, retention, and disposal |  | Content sheet 5-5 | Presentation 5, Slides 25-28 |  |
| 7 | 15 min | Sample transport |  | Content sheet 5-6 | Presentation 5, Slides 29-37 |  |
| 8 | 5 min | Summary |  | Content sheet 5-7 | Presentation 5, Slides 38-40 |  |
| Optional Material on the CD-ROM |  |  |  |  |  |  |
| Collection and preservation |  |  |  | Annex 5-E Annex 5-F |  |  |
| Sample transport |  |  |  | Annex 5-G |  |  |

Material/Equipment Checklist

- PowerPoint slides or transparencies

- Overhead projector or computer with an LCD projector

- Prepared flipchart, white board, or chalk board

- Paper cards, markers, and tape

- Additional handouts as required.